# Risk Assessment – 007 Laser Shooting Game Hire (HSE/AAIAC Style)

## Activity Description

Delivery, setup, testing, supervised operation and dismantling of the 007 Laser Shooting Game for temporary hire at events. Players use infrared laser guns to aim at electronic targets in a controlled game area.

## Persons at Risk

Staff, hirers, operators, participants, spectators and members of the public.

| Hazard | Persons at Risk | Initial Risk | Control Measures | Residual Risk |
| --- | --- | --- | --- | --- |
| Manual handling of equipment | Staff | Medium | Use safe lifting techniques and transport aids. | Low |
| Electrical fault or damaged cables | Staff/Public | High | PAT tested equipment, RCD protection and pre-use inspection. | Low |
| Trip hazards from cables | All | Medium | Protect leads with cable covers and keep walkways clear. | Low |
| Unsafe aiming of laser guns | Participants | Medium | Safety briefing; aim only at designated targets. | Low |
| Player collision while moving | Participants | Medium | Clearly mark playing positions and supervise activity. | Low |
| Equipment malfunction | All | Medium | Function test before each hire; isolate faulty equipment. | Low |
| Unauthorised access to play area | Public | Medium | Maintain barriers and operator control. | Low |

## Operational Controls

Set up the attraction on a firm, level surface with a clearly defined shooting area and safe spectator zone.

Test all laser guns, electronic targets, scoring systems and sound effects before opening.

Protect and secure all power leads using cable covers and keep them away from player movement areas.

Allow only the manufacturer's recommended number of players to participate at one time.

Brief players to aim only at designated electronic targets and never at other people.

Players must remain within the designated shooting position unless instructed otherwise.

Maintain continuous operator supervision throughout play.

Stop the game immediately if unsafe behaviour, damaged equipment or faults are identified.

## Emergency Procedures

Stop the game immediately if unsafe behaviour, damaged equipment or faults are identified.

Isolate the electrical supply before inspecting the equipment.

Provide first aid and contact emergency services if required.

Remove defective equipment from service until repaired.

Record all accidents, incidents and near misses.

## Inspection Checklist

Cabinet stable and level.

Laser guns and triggers operate correctly.

Targets, sensors and scoring display operate correctly.

Power cable and plug undamaged.

PAT label current where applicable.

No loose fixings, exposed wiring or sharp edges.

Playing area free from slip and trip hazards.

## Training Requirements

Safe delivery, installation and dismantling.

Manual handling techniques.

Electrical safety and cable management.

Pre-use inspections and functional testing.

Participant briefing and supervision.

Emergency isolation and accident reporting.