Risk Assessment – Inflatable Climbing Wall Hire (HSE/AAIAC Style)

# Activity Description

Delivery, setup, supervised operation and dismantling of inflatable climbing wall equipment for temporary hire at events.

# Persons at Risk

Staff, hirers, operators, participants, spectators and members of the public.

| Hazard | Persons at Risk | Initial Risk | Control Measures | Residual Risk |
| --- | --- | --- | --- | --- |
| Manual handling of inflatable and equipment | Staff | Medium | Use safe lifting techniques and team lifts where required; secure components during transport and setup. | Low |
| Inflatable instability / inadequate anchoring | Participants/Spectators | High | Set up on suitable firm, level ground; use specified anchoring; inspect before use and stop if movement occurs. | Low |
| Participant fall from climbing wall | Participants | High | Operate strictly to manufacturer's instructions; inspect climbing surface, harnesses, ropes, anchors and landing arrangements; competent supervision at all times. | Low |
| Harness / rope / attachment failure | Participants | High | Inspect all climbing and safety equipment before each session; use only approved components; remove defective equipment from service. | Low |
| Incorrect participant age / size / ability | Participants | High | Apply manufacturer's age, height, weight and ability limits; assess suitability before participation. | Low |
| Collision / fall into another participant | Participants | Medium | Control one climber per route where required; maintain clear landing area; supervise access and exit. | Low |
| Climbing holds / structure damaged | Participants/Staff | High | Inspect holds, seams, structure and attachment points before use; stop operation if damage or excessive wear is identified. | Low |
| Incorrect use / deliberate misuse | Participants | High | Provide safety briefing; prohibit jumping, swinging, climbing outside designated routes and unsafe behaviour. | Low |
| Trips, slips and falls around activity | All | Medium | Maintain clear access, landing and queue areas; keep surrounding surface clean and dry. | Low |
| Overcrowding / spectator access | Participants/Spectators | Medium | Control participant numbers and queues; maintain exclusion zones around climbing and landing areas. | Low |
| Emergency access obstructed | All | Medium | Position the climbing wall so emergency routes, exits and venue access remain clear. | Low |

# Operational Controls

Set up the Inflatable Climbing Wall activity on suitable ground/surface with adequate clearance and access.

Inspect equipment before each use and provide a clear safety briefing before participation.

Control participant numbers and maintain clearly defined playing, access and spectator areas.

Prohibit running, pushing, climbing, deliberate misuse and other unsafe behaviour.

Follow manufacturer operating, participant, anchoring and weather requirements where applicable.

Stop the activity immediately if equipment is damaged, unstable or conditions become unsafe.

# Emergency Procedures

Stop the Inflatable Climbing Wall activity immediately following any incident or unsafe condition.

Ensure participants leave the active area safely and keep emergency routes clear.

Provide first aid where required and contact emergency services if necessary.

Remove damaged equipment from service and record all accidents and near misses.

# Inspection Checklist

Equipment structure and components stable, secure and free from damage.

Activity surface, access and landing areas clean and free from defects or trip hazards.

Access, exit and spectator areas clearly defined.

Emergency routes and venue access remain clear.

Participant capacity, supervision and restrictions clearly established.

# Training Requirements

Staff trained in safe Inflatable Climbing Wall operation, equipment inspection, participant briefing, supervision, crowd/access control, emergency procedures and incident response.