Risk Assessment – Inflatable Zip Wire Hire (HSE/AAIAC Style)

# Activity Description

Delivery, setup, supervised operation and dismantling of inflatable zip wire equipment for temporary hire at events.

# Persons at Risk

Staff, hirers, operators, participants, spectators and members of the public.

| Hazard | Persons at Risk | Initial Risk | Control Measures | Residual Risk |
| --- | --- | --- | --- | --- |
| Manual handling of inflatable and equipment | Staff | Medium | Use safe lifting techniques and team lifts where required; secure all components during transport and setup. | Low |
| Inflatable instability / inadequate anchoring | Participants/Spectators | High | Set up on suitable firm, level ground; use specified anchoring; inspect before use and stop if movement occurs. | Low |
| Fall / injury during zip wire use | Participants | High | Operate strictly to manufacturer's instructions; inspect carriage, cable/track, harness/restraints and landing area; one user at a time where required; competent supervision. | Low |
| Incorrect participant size / ability | Participants | High | Apply manufacturer age, height, weight and ability restrictions; assess suitability before participation. | Low |
| Collision at landing area | Participants/Spectators | High | Keep landing and exit zone clear; ensure previous participant has exited before next use. | Low |
| Equipment or restraint failure | Participants | High | Pre-use inspection of all relevant components; remove equipment from service if any defect, wear or damage is found. | Low |
| Trips, slips and falls | All | Medium | Maintain clear access and landing areas; remove loose equipment and hazards promptly. | Low |
| Incorrect use / deliberate misuse | Participants | High | Provide safety briefing; prohibit unsafe loading, swinging, climbing or interference with equipment. | Low |
| Overcrowding / spectator access | Participants/Spectators | Medium | Control queues and numbers; maintain clear exclusion zones around launch and landing areas. | Low |
| Electrical equipment / blower | Staff/Participants | High | Inspect blower, cables and connections; route cables safely and keep electrical connections dry. | Low |
| Emergency access obstructed | All | Medium | Maintain clear emergency routes, exits and access around the attraction. | Low |

# Operational Controls

Set up the Inflatable Zip Wire activity on suitable ground/surface with adequate clearance and access.

Inspect equipment before each use and provide a clear safety briefing before participation.

Control participant numbers and maintain clearly defined playing, access and spectator areas.

Prohibit running, pushing, climbing, deliberate misuse and other unsafe behaviour.

Follow manufacturer operating, participant, anchoring and weather requirements where applicable.

Stop the activity immediately if equipment is damaged, unstable or conditions become unsafe.

# Emergency Procedures

Stop the Inflatable Zip Wire activity immediately following any incident or unsafe condition.

Ensure participants leave the active area safely and keep emergency routes clear.

Provide first aid where required and contact emergency services if necessary.

Remove damaged equipment from service and record all accidents and near misses.

# Inspection Checklist

Equipment structure and components stable, secure and free from damage.

Activity surface, access and landing areas clean and free from defects or trip hazards.

Access, exit and spectator areas clearly defined.

Emergency routes and venue access remain clear.

Participant capacity, supervision and restrictions clearly established.

# Training Requirements

Staff trained in safe Inflatable Zip Wire operation, equipment inspection, participant briefing, supervision, crowd/access control, emergency procedures and incident response.