# Comprehensive Risk Assessment – Petrol Go Kart Hire (Public Events)

## Activity Details

Activity: Petrol Go Kart Hire
Suitable for: Schools (where permitted), Corporate Events, Public Events, Private Hire
Operators: Trained staff only
Review: Annually or following any incident

| Hazard | Control Measures | L | S | Risk | Further Action |
| --- | --- | --- | --- | --- | --- |
| Transport & unloading | Team lifting, loading ramps, exclusion zone during unloading. | 1 | 3 | 3 | Staff training |
| Track construction | Level surface, impact barriers, marshal gates, secure fencing. | 1 | 5 | 5 | Pre-opening inspection |
| Mechanical failure | Daily checks of steering, brakes, throttle, tyres, chain, fuel lines and kill switch. | 1 | 5 | 5 | Maintenance log |
| Fuel handling | Refuel only when engine is off and cool. Store petrol in approved containers away from ignition sources. | 1 | 5 | 5 | Fuel procedure |
| Fire & explosion | Fire extinguishers (foam/CO₂) at pit and refuelling area. No smoking. | 1 | 5 | 5 | Emergency drills |
| Exhaust fumes | Operate outdoors or in adequately ventilated areas only. | 1 | 5 | 5 | Monitor ventilation |
| Noise exposure | Operators wear hearing protection where required. Participants briefed. | 2 | 3 | 6 | Noise assessment |
| Collision | Speed limits, marshals, one-way system, safety briefing. | 2 | 4 | 8 | Continuous supervision |
| Pedestrian intrusion | Barriered circuit and controlled access. | 1 | 5 | 5 | Marshal supervision |
| Driver suitability | Age, height, weight and medical restrictions enforced. | 2 | 4 | 8 | Operator checks |
| PPE | Helmets mandatory; neck collars/gloves where required. | 1 | 4 | 4 | Inspect PPE |
| Weather | Suspend operation during heavy rain, flooding or unsafe conditions. | 1 | 5 | 5 | Weather monitoring |
| Environmental spill | Spill kit available; fuel spills cleaned immediately. | 1 | 4 | 4 | Spill reporting |
| Emergency | Emergency stop, isolate track, first aid, contact emergency services if required. | 1 | 5 | 5 | Incident report |

## Daily Pre-Opening Checklist

Inspect all karts.

Check brakes, steering and throttle return.

Check tyres and fuel lines.

Test kill switches.

Inspect barriers and track.

Verify fire extinguishers, spill kit and first aid kit.

Complete inspection log.

## Operator Responsibilities

Conduct driver safety briefing.

Check PPE fit.

Monitor driving behaviour.

Control refuelling.

Stop activity if unsafe.

Record incidents.

## Emergency Action Plan

Stop all karts immediately.

Activate emergency procedures.

Isolate fuel sources.

Use extinguisher only if safe.

Provide first aid.

Call 999 where required.

Secure incident area.

## Applicable UK Legislation & Guidance

Health and Safety at Work etc. Act 1974

Management of Health and Safety at Work Regulations 1999

Provision and Use of Work Equipment Regulations (PUWER) 1998

Control of Substances Hazardous to Health (COSHH) Regulations 2002 (fuel handling)

Dangerous Substances and Explosive Atmospheres Regulations (DSEAR) 2002

Electricity at Work Regulations 1989 (where electrical equipment is used)

Manufacturer's operating manual