Risk Assessment – Human Gyroscope Hire (HSE/AAIAC Style)

# Activity Description

Delivery, setup, supervised operation, participant loading/unloading and dismantling of a Human Gyroscope amusement activity for temporary hire at events.

# Persons at Risk

Staff, ride operators, participants, spectators, hirers and members of the public.

| Hazard | Persons at Risk | Initial Risk | Control Measures | Residual Risk |
| --- | --- | --- | --- | --- |
| Manual handling of gyroscope and equipment | Staff | Medium | Use safe lifting techniques and team lifts/mechanical aids where required; secure the gyroscope, frame and accessories during transport, setup and dismantling. | Low |
| Gyroscope instability / incorrect setup | Participants/Spectators | High | Set up on suitable firm, level ground; assemble and secure in accordance with manufacturer instructions; complete pre-use inspection before operation. | Low |
| Participant falling / loss of balance | Participants | High | Use the manufacturer's approved restraint/harness system; check participant is correctly secured; operate within specified limits; competent supervision throughout. | Low |
| Hair entrapment in rotating equipment | Participants/Staff | High | Long hair must be securely tied back and kept clear of all moving parts; loose clothing, scarves, jewellery and other items that could become caught must be removed or secured. Stop the ride immediately if hair or clothing becomes caught. Do not operate until the participant is safely released and the equipment is checked. | Low |
| Clothing / loose items becoming entangled | Participants | High | Check clothing, footwear, jewellery and personal items before entry; secure or remove anything that could catch in moving parts; keep hands and body clear of mechanisms. | Low |
| Collision with gyroscope/frame | Participants | High | Maintain required clearance and exclusion zone; prohibit spectators entering the operating area; supervise continuously. | Low |
| Incorrect use / deliberate misuse | Participants | High | Provide a clear safety briefing; prohibit climbing onto the frame, standing, unsafe movement or interference with restraints and controls. | Low |
| Mechanical / restraint failure | Participants | High | Inspect frame, bearings, restraints, harnesses, locking mechanisms and safety devices before each use; remove defective equipment from service. | Low |
| Unsuitable participant age / size / ability | Participants | High | Apply manufacturer's age, height, weight and ability restrictions; assess suitability before participation. | Low |
| Trips / slips around ride | All | Medium | Maintain clean, dry and unobstructed access and operating areas; control queues and keep loose equipment away from walkways. | Low |
| Overcrowding / spectator access | Participants/Spectators | Medium | Control participant numbers and queues; maintain a clearly defined operating and spectator exclusion area. | Low |
| Unexpected movement / excessive rotation | Participants | High | Operate only within manufacturer's speed and movement limits; competent operator controls the ride and stops it if abnormal movement occurs. | Low |
| Weather / outdoor conditions | All | High | Follow manufacturer's weather and wind limits; secure equipment correctly; stop operation in unsuitable conditions. | Low |
| Emergency stop / participant evacuation | All | High | Operator trained in emergency stop and release procedures; keep emergency access clear; follow manufacturer's emergency procedure. | Low |
| Emergency access obstructed | All | Medium | Position the gyroscope so emergency routes, exits and venue access remain clear at all times. | Low |

# Operational Controls

Set up the Human Gyroscope on suitable firm, level ground with adequate clearance and a controlled exclusion zone.

Complete documented pre-use checks of the frame, rotating mechanisms, restraints/harness, locking systems and safety devices.

Check participant age, height, weight and suitability before fitting the restraint/harness.

Secure long hair and remove or secure loose clothing, scarves, jewellery and personal items before operation.

Give a clear safety briefing and maintain competent active supervision throughout the ride.

Operate only within the manufacturer's specified capacity, speed and operating limits; stop immediately if any unsafe condition develops.

# Emergency Procedures

Stop the Human Gyroscope immediately following any incident, equipment fault or unsafe condition.

Use the manufacturer's emergency stop, release and participant evacuation procedure.

If hair, clothing or an item becomes entangled, stop movement immediately and do not restart until the participant is safely released and the equipment has been checked.

Provide first aid where required and contact emergency services if necessary.

Isolate defective equipment and record all accidents, incidents and near misses.

# Inspection Checklist

Frame, rotating components, bearings and structural parts secure and free from damage.

Restraints/harnesses, locks and participant securing systems checked and functioning.

Emergency stop and safety devices checked before operation.

Hair, clothing, jewellery and loose-item entrapment risks controlled before every ride.

Operating area, exclusion zone and emergency access clear.

Participant restrictions and operating limits clearly established.

# Training Requirements

Operators trained and competent in Human Gyroscope operation, pre-use inspection, restraint/harness checks, hair and clothing entrapment prevention, participant briefing, supervision, emergency stop/evacuation procedures, manufacturer operating limits and incident reporting.