# Risk Assessment – Inflatable Axe Throwing Hire (Public Events)

Persons Involved: Trained Operators, Event Staff
Also Present: Participants, Spectators, Venue Staff
Equipment: Inflatable axe throwing game, inflatable target board, foam or Velcro axes, blower, anchor stakes or sandbags, RCD-protected extension leads, barriers, signage
Location: Various Indoor and Outdoor Public Event Locations

| Hazard | Existing Controls | Likelihood | Severity | Risk | Further Action |
| --- | --- | --- | --- | --- | --- |
| Manual handling | Use safe lifting techniques and team lifts where required. | 1 | 3 | 3 | Staff training. |
| Installation | Installed by trained staff following manufacturer instructions. | 1 | 5 | 5 | Pre-opening inspection. |
| Anchoring | Inflatable secured with approved stakes or ballast to manufacturer specification. | 1 | 5 | 5 | Check throughout event. |
| Blower failure | Continuous blower operation monitored. Close game immediately if blower stops. | 1 | 5 | 5 | Inspect blower and power supply before reopening. |
| Inflation/Deflation | No users permitted during inflation or deflation. | 1 | 5 | 5 | Operator control only. |
| Wind & Weather | Do not operate above manufacturer wind limits (typically 24 mph / 38 km/h). Suspend use during heavy rain or thunderstorms. | 1 | 5 | 5 | Continuous weather monitoring. |
| Trip hazards | Secure cables and keep access routes clear. | 1 | 2 | 2 | Routine housekeeping inspections. |
| Misthrown axes | Only foam or Velcro axes supplied. Participants throw from designated line. | 2 | 3 | 6 | Operator supervision. |
| Participant struck by axe | One thrower at a time unless game design permits otherwise. Safe waiting area provided. | 2 | 3 | 6 | Enforce throwing rules. |
| Spectator safety | Barriered viewing area maintained behind throwing line. | 1 | 3 | 3 | Marshal if required. |
| Horseplay / misuse | No running, wrestling, climbing, or throwing axes outside the designated area. | 2 | 4 | 8 | Immediate removal from activity. |
| Children | Minimum age requirements applied where appropriate. Adult supervision for younger participants. | 2 | 3 | 6 | Display age restrictions. |
| Electrical safety | PAT-tested blower and RCD protection used. Cables protected from damage. | 1 | 5 | 5 | Daily electrical inspection. |
| HSE guidance | Operate in accordance with HSE guidance for inflatable play equipment and manufacturer operating instructions. | 1 | 4 | 4 | Annual review and staff refresher training. |
| Emergency | Stop activity immediately if unsafe. Provide first aid and call 999 if required. | 1 | 5 | 5 | Record incidents and quarantine faulty equipment. |

## Additional Control Measures

Only trained operators supervise the activity.

A full participant safety briefing is given before play begins.

Throwing line and exclusion zones are clearly marked.

Foam or Velcro axes are inspected before every session.

Damaged axes, targets, or inflatable components are removed from service immediately.

First aid kit and emergency contact details are available on site.

Capacity limits are monitored to prevent overcrowding around the inflatable.

The activity is operated in accordance with the manufacturer's operating manual and event safety procedures.